# During a call

This section describes tasks you can perform when you are currently engaged in a call.   Soft key and function key names are in Bold.  Phone number means to dial a phone number or an extension number.

| Task | Key Sequence |
| --- | --- |
| Create conference call with call on hold | Features soft key + Conference + new call + Features + Conference |
| End call | End key |
| Hold call | Hold soft key |
| Park current call | Features soft key + Navigation keys + Park + orbit number + Features soft key + Park |
| Split conference into 2 calls on hold | Features soft key + Split |
| Transfer call after consulting | Features soft key + Transfer + phone number + Features soft key + Transfer |
| Transfer call without consulting (blind transfer) | Blind Transfer soft key + phone number + Start key |
| Transfer call to another user's voicemail | Blind Transfer soft key + phone number +  |

# Other tasks:

# This section describes tasks you can perform when not currently engaged in a call.  Soft key and function key names are in Bold.  Phone number means to dial a phone number or an extension number.

| Task | Key Sequence |
| --- | --- |
| Access voice mail | Navigation keys + Messages + OK key |
| Answer call | Start key |
| Create conference call | Phone number + Features soft key + Conference + phone number + Features soft key + Conference |
| End call | End key |
| Resume call | Call on hold + Resume soft key |
| List all parked calls |  + 904 + Start key |
| List one parked call |  + 904 + orbit number + Start key |
| Log in or out of a phone | To log in:  + Dial + 98 + extension + password + # + 1 To log out:  + Dial + 98 + extension + password + # + 2 Menu options available on the 98 menu include: 1 - Log in, 2 - Log out, 3 - Log in to a different station, 4 - Change your status, 5 - record your agent greeting and 6 - Log in or out of queues. |
| Page |  + 901 + zone number + Start key |
| Pick up held or alerting call (on a station to which you have access) | Start key +  + pause + 95 + extension |
| Pick up group call | Start key + Features soft key + Pickup + Features soft key + Group |
| Pick up parked call | Start key + Features soft key + Pickup + orbit number + Retrieve |
| Place call | Phone number + Start key or Start key + Phone number or Features soft key + Lines + OK key + phone number + Start key or Features soft key + Lines + OK key + Navigation keys to contact name or number + OK key |

### SpectraLink® 
8400 WiFi Series

### Quick Reference

These instructions highlight SpectraLink phone features. See your SpectraLink 8400 Series Quick Start Guide for more detailed instructions on using the home screen, soft keys, and handset features.

Application firmware requirements for each model are listed on the SIP IP Phones information page available at the Interactive Intelligence Testlab website, http://testlab.inin.com.

Interactive Intelligence, Inc.

7601 Interactive Way

Indianapolis, IN 46278

Telephone/Fax: (317) 872-3000

www.ININ.com

DC-012-4.0-QRP6	Revised 1/18/2016

# SpectraLink Keys

| Icon | Function |
| --- | --- |
|  | Start key |
|  | End key |
|  | OK key |
|  | Navigate left key |
|  | Navigate right key |
|  | Navigate up key |
|  | Navigate down key |
|  | Soft keys |
|  | Voicemail icon |
|  | Settings icon |

# Access voicemail

Received messages are indicated by the Voicemail icon in the status bar.

1.	Press the Navigation keys until the Messages icon appears on the LCD display.

2.	Press the OK key.

3.	Press the OK key to open the Voicemail… menu.

4.	The LCD display indicates number of Urgent, New, and Old messages, and their importance. Use the Navigation keys to highlight the voicemail message 
you want to hear.

5.	Press Connect to initiate retrieval from the Message Center, then follow voice prompts to listen to messages.

Or press the Clear soft key to clear Voicemail messages.

Voicemail is an optional feature that requires both Interaction Administrator and phone configuration. Contact your system administrator.

# Create a conference call

1.	Call the first party.

2.	Press the Features soft key and use the Navigation keys to highlight Conference.

3.	Press the OK key to get a new line and place the first call on hold.

4.	Dial the second party.

5.	When second party answers, press the Features soft key and use the Navigation keys to highlight Conference.

6.	Press the OK key to join both calls in a conference.

## Join a conference call

Press the Features soft key, select Join, and press the OK key to create a conference call at any time between an active call and a call on hold on the same line.

## Split a conference call

1.	After you establish a conference call, highlight the active conference call and press the Features soft key.

2.	Use the Navigation keys to highlight Split and press the OK key to place both calls on hold.

# Distinctive ring

If your phone is a Managed IP Phone, your CIC administrator can configure distinctive ring tones for incoming Internal, External, and Direct Dialed calls. This overrides Ring Type settings you make directly on your phone.

# Do Not Disturb (DND)

The Enable/Disable soft key appears when you access Feature Settings from the Settings icon on the LCD display. If your phone is a Managed IP Phone, your CIC administrator can enable DND synchronization between your phone setting and your CIC user status.

1.	Use the Navigation keys to display the Settings icon on the LCD display, and then press the OK key.

2.	Use the Navigation keys to highlight Feature Settings… and press the OK key. Highlight Do Not Disturb and press the OK key.  The Enable/Disable soft key appears.

2.	To prevent the phone from ringing on incoming calls and to set your CIC user status to Do Not Disturb, select the Enable soft key.

3.	To turn DND status off on the phone and to set your CIC user status to your last available status or other selected status, select the Disable soft key.

Refer to CIC client help for details.

# Hold and resume a call

The Hold/Resume soft key appears during active calls.

During a call, press the Hold soft key to place the call on hold. Press the Resume soft key to retrieve the call.

Note: If multiple calls are on hold on a single line key, use the Navigation keys to switch between them, then press Resume.

# Page a zone

1.	Dial  + 901 + zone number and press the Start key.  (The phone beeps to prompt you to start talking.)

2.	Complete page and hang up.

Tip: Zone numbers are station extensions, station group extensions, user extensions, or workgroup extensions.

# Park a call

You can park your currently active call in a specific orbit. This places the call on hold and removes it from your station.

1.	During a call, press the Features soft key and then use the Navigation keys to highlight Park.

2.	Press the OK key.

3.	Enter an orbit number.

4.	Press the Features soft key again and then use the Navigation keys to highlight Park.

5.	Press the OK key.

Note: You can use any single digit to 9 digit number, and 10 digit numbers up to 4294967294.  Avoid orbit numbers starting with 0 (zero).  If the orbit number is already in use, you remain connected to the caller. Repeat this procedure and choose another orbit number.

## Pick up a parked call

You can pick up a parked call from any other station.

1.	Press the Start key.

2.	Press the Features soft key and use the Navigation keys to highlight Pickup.

3.	Press the OK key.

4.	Enter the orbit number and press the Features soft key again.

5.	Use the Navigation keys to highlight the Retrieve feature and press the OK key again.

## List parked calls

You can hear the details of parked calls: orbit number, who called, and how long the call has been on hold. Do one of the following:

- For all parked calls: Press  + 904 + the Start soft key.

- For one parked call: Press  + 904 + orbit number + Start soft key.

# Pick up a group call

You can answer a call that is ringing on any extension in your group by doing one of the following:

1.	Press the Start key.

2.	Press the Features soft key and use the Navigation keys to highlight Pickup.

3.	Press the OK key.

4.	Press the Features soft key and use the Navigation keys to highlight Group.

5.	Press the OK key.

# Place a call

Use any of the following methods for placing a call:

- Dial a number and press the Start key.

- Press the Start key and dial a number.

- Press the Features soft key and select Lines. Press the OK key, dial a number, and press the Start key.

- Press the Features soft key and select Lines. Press the OK key, navigate to a contact or number, and press the Start key.

# Transfer a call after consulting

Before you transfer a call, you can consult the party to whom you are transferring the call or you can just transfer the call (blind transfer). To transfer a call after consulting the recipient:

1.	During a call, press the Features soft key and use the Navigation keys to highlight Transfer.

2.	Press the OK key.

3.	Dial the number to which you want to transfer the call.

4.	After consulting the party to whom you are transferring the call, press the Features soft key again.

5.	Use the Navigation keys to highlight transfer and press the OK key to complete the transfer operation.

# Transfer a call without consulting

You can transfer a call without consulting the party to whom you are transferring the call (blind transfer).

1.	During a call, press the Blind Transfer soft key.

2.	Dial the number to which you want to transfer the call.

3.	Press the OK key and then press the Start key.

# Transfer a call to voice mail

You leave a message for another CIC user or transfer your current call directly to another CIC user's voice mail. Do one of the following:

- Press  after dialing a CIC user's extension number to leave a voice mail message.

- To transfer a call to another CIC's user's voice mail, press the Blind Transfer soft key, dial the extension number and press .

# Two Way Intercom Page

Your phone can be configured to work as intercom.‡

1.	Get a dial tone.

2.	Dial  + 900 + extension number and press the Start key.

- If the number dialed is not busy, the intercom call is connected and you can start talking. (The receiving phone may play a tone to alert the person called.)

- If the receiving phone is busy, then the call can be sent to voicemail, transferred, or disconnected like any other call.

3.	Complete the conversation and hang up or press the End key.

‡ Intercom paging is an optional feature that must configured for both the sending and receiving phones, users and CIC stations. It may not be supported on your phone. Contact your system administrator.